| # GOLF-RPW01 |  |  |
| --- | --- | --- |

| # OPTIC OPTIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # 110x20˚ | # 30˚ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| # SPECIFICATIONS AVAILABLE FOR RPW01 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Sample Part Number: RPW011-1W-1K-K30-D24-110X20D-CL-N-N-HS |  |  |  |  |  |  |  |  |  |
| # FIXTURE TYPE | # WATTAGE | # LEDS | # CCT. | # INPUT VOLTAGE | # BEAM ANGLE | # CONTROL | # ANTI-GLARE ACCESSORIES | # BRACKET | # OPTIONS/ CUSTOMIZATION* |
| # RPW011 | # 1W # 3W | # 1K LEDs x1 | # K30 3000K # RGBWH RGBW 4in1 (3W/9W only) | # D24 24VDC | # 110x20D 110x20˚ # 30D 30˚ | # CL On-Off # DMX DMX1 | # N None | # N None | # HS RAL9001 # RAL# Custom RAL# |
| # RPW012 | # 6W # 9W | # 3K LEDs x3 |  |  |  |  |  |  |  |

| SPECIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| ELECTRICAL | Input Voltage | 24VDC |  |  |
|  | Typical Power Consumption | Max.1W ±10% ¦ Max.3W ±10% (@RPW011); Max.6W ±10% ¦ Max.9W ±10% (@RPW012) |  |  |
| OPTICAL | Light Source | High Power LED |  |  |
|  | Color Variants | Mono: 3000K (Ra.>70) |  |  |
|  |  | Color: RGBW(4in1) |  |  |
|  | Beam Angle (FWHM) | Mono: 110x20˚¦ 30˚(LEDs x1, LEDs x3) |  |  |
|  |  | RGBW(4in1): 110x20˚¦ 30˚(LEDs x1, LEDs x3) |  |  |
|  | Efficacy (lm/W) | Mono: 50~60 (±10%) |  |  |
| CONTROL | Power Supply | Integrated |  |  |
|  | Operating Modes | On-Off; DMX |  |  |
|  | Control System | DMX512 Controllers |  |  |
|  | Settings / Addressing | DMX Communicator |  |
| PHYSICAL | Length x Width x Height ¦ Weight | 104mm(L) x 104mm(W) x 35mm(H) ¦ 0.3kg @RPW011 |  |  |
|  |  | 143mm(L) x 143mm(W) x 45mm(H) ¦ 0.5kg @RPW012 |  |  |
|  | Housing | Body: Die-cast Aluminum ADC12 |  |  |
|  |  | Cover: Tempered Glass |  |  |
|  | Finish Options | Standard Color | Graphite Black RAL 9011 |  |
|  |  | Cost Option | Custom RAL # |  |
|  | Cables / Connections | ON/OFF | DMX |  |
|  |  | 2 Core Cable with Male/Female Connector (2x1.0 mm² Cable Power IN/OUT) | 5 Core Cable with Male/Female Connector (2x1.0 mm² Power + 3x0.3 mm² Signal IN/OUT) |  |
|  | Mounting Method | Surface Mounting |  |  |
|  | Adjustable | No |  |  |
|  | Protection Factor | Class III |  |  |
|  | IP ¦ IK Rating | IP67 ¦ IK10 |  |  |
|  | Operating Ambient Temperature | −40°C ~ +50°C |  |  |
| CERTIFICATION | Listings |  |  |  |

| PHOTOMETRIC DATA |  |  |  |
| --- | --- | --- | --- |
| Beam Angle (FWHM): 110 x 20˚ | Beam Angle (FWHM): 30˚ |  |  |

| DIMENSIONS |  |
| --- | --- |
| @RPW011 | @RPW012 |